ADI VSM Study Watch Driver Reference Guide

REV 1.0.0,FEB 2021

Table of Contents

1 Introduction	5

1.1 Scope	5

1.2 Organization of this Guide	5

1.3 Acronyms	5

1.4 References	5

1.5 Additional Information	5

2 Specifications	6

2.1 Version Information	6

3 ADPD4100 API Guide	7

3.1 API Functions	7

3.1.1 Adpd400xDrvOpenDriver	7

3.1.2 Adpd400xDrvCloseDriver	7

3.1.3 Adpd400xDrvGetComMode	8

3.1.4 Adpd400xDrvRegWrite	8

3.1.5 Adpd400xDrvRegRead	8

3.1.6 Adpd400xDrvRegRead32B	9

3.1.7 Adpd400xDrvSetOperationMode	10

3.1.8 Adpd400xDrvSetOperationPause	10

3.1.9 Adpd400xDrvSlotSetup	10

3.1.10 Adpd400xDrvSlotSetActive	11

3.1.11 Adpd400xDrvDataReadyCallback	12

3.1.12 Adpd400xISR	12

3.1.13 Adpd400xDrvSetParameter	12

3.1.14 Adpd400xDrvGetParameter	13

3.1.15 Adpd400xDrvReadFifoData	14

3.1.16 Adpd400xDrvReadRegData	15

3.1.17 Adpd400xDrvSetLedCurrent	15

3.1.18 Adpd400xDrvGetLedCurrent	16

3.1.19 Adpd400xDrvSoftReset	17

3.1.20 _Adpd400xDrvInit	17

3.1.21 _Adpd400xDrvSetInterrupt	17

3.1.22 _Adpd400xDrvSetIdleMode	18

3.1.23 _Adpd400xDrvGetSlotInfo	18

3.1.24 _Adpd400xDrvGetDataOuputRate	18

3.1.25 _Adpd400xDrvSlotSaveCurrentSetting	19

3.1.26 _Adpd400xDrvSlotApplyPreviousSetting	19

3.1.27 _Adpd400xDrvSlotApplySkipSetting	20

3.1.28 _Adpd400xDrvSetSlotSize	20

3.1.29 _FifoLevel	20

3.2 API Data Types	21

3.2.1 adpd400xDrv_slot_t	21

3.3 Enums	22

3.3.1 ADPD400xDrv_Operation_Mode_t	22

3.3.2 ADPD400XDrv_FIFO_SIZE_t	22

3.3.3 Adpd400xCommandStruct_t	22

3.3.4 ADPDDrvCl_SignalDark_t	23

3.3.5 ADPD400xDrv_SlotNum_t	23

3.3.6 ADPD400xDrv_LedId_t	23

3.3.7 Adpd400xComMode_t	24

List of Figures

No table of figures entries found.

List of Tables

No table of figures entries found.
Copyright, Disclaimer & Trademark Statements

Copyright Information

Copyright (c) 2021 Analog Devices, Inc.  All Rights Reserved.  This documentation is proprietary and confidential to Analog Devices, Inc. and its licensors.  This document may not be reproduced in any form without prior, express consent from Analog Devices, Inc.

Disclaimer

Analog Devices, Inc. (“Analog Devices”) reserves the right to change this product without prior notice.  Information furnished by Analog Devices is believed to be accurate and reliable.  However, no responsibility is assumed by Analog Devices for its use; nor for any infringement of patents or other rights of third parties which may result from its use.  No license is granted by implication or otherwise under the patent or other rights of Analog Devices

Trademark and Service Mark Notice

Analog Devices, the Analog Devices logo, Blackfin, SHARC, TigerSHARC, CrossCore, VisualDSP, VisualDSP++, EZ-KIT Lite, EZ-Extender, SigmaStudio and Collaborative are the exclusive trademarks and/or registered trademarks of Analog Devices, Inc (“Analog Devices”).

All other brand and product names are trademarks or service marks of their respective owners.

Analog Devices’ Trademarks and Service Marks may not be used without the express written consent of Analog Devices, such consent only to be provided in a separate written agreement signed by Analog Devices.  Subject to the foregoing, such Trademarks and Service Marks must be used according to Analog Devices’ Trademark Usage guidelines. Any licensee wishing to use Analog Devices’ Trademarks and Service Marks must obtain and follow these guidelines for the specific marks at issue.

# Introduction

This is a living document describing the various device drivers used on ADI VSM Study Watch.

## Scope

The API of the drivers, to enable software developers to use in a C application, and for system integrators to build it into a complete system. Developers are assumed to be familiar with the ADI sensors/accelerometer.

## Organization of this Guide

Section 1: this section contains the introduction

Section 2: lists specifications of the devices

Section 3: ADPD4100 driver API guide

Section 4: ADXL362 driver API guide

## Acronyms

| ADI | Analog Devices Inc. |
| --- | --- |
| API | Application Program Interface |
| ADPD | Analog Devices Photo Diode Sensor |

## References

- ADPD4100 Data sheet

- ADXL362 Data sheet

## Additional Information

For more information on the latest ADI processors, sensors, silicon errata, code examples,

development tools, system services and devices drivers, technical support and any other

additional information, please email healthcare-support@analog.com, or visit our website at www.analog.com/processors.

# Specifications

The drivers covered in this document include

- ADPD4100 – This is the latest multimodal sensor front end, which can stimulate up to eight LEDs and measuring the return signal on up to eight separate current inputs. It has twelve time slots which enables 12 separate measurements per sampling period. The driver utilizes an SPI interface for data output and functional configuration.

- ADXL362 – This is the ultralow power, 3-axis accelerometer which provides 12-bit output resolution supporting measurement ranges of +/-2g, +/-4g and +/-8g.

## Version Information

This document describes release 1.0.0 of the ADI VSM Study Watch device driver document.

# ADPD4100 API Guide

The driver comes with the following header files:

- adpd400x_drv.h – This includes the API definitions, macros, enums and structures used in driver.

- adpd400x_reg.h – This file contains the register addresses and bit definitions of each register.

## API Functions

### Adpd400xDrvOpenDriver

Prototype

int16_t Adpd400xDrvOpenDriver(void;

Description

This function sets up the interface lines, initialization of driver and interrupt mode as FIFO. It can check whether the interface is SPI or I2c and accordingly set the communication mode.

Parameters

None.

Return value

ADPD400xDrv_SUCCESS – Driver open successful

ADPD400xDrv_ERROR – Error in opening the driver

### Adpd400xDrvCloseDriver

Prototype

int16_t Adpd400xDrvCloseDriver(void);

Description

Sets up the device in idle mode and closes the driver.

Parameters

None

Return value

ADPD400xDrv_SUCCESS – Driver close successful

ADPD400xDrv_ERROR – Error in closing the driver

### Adpd400xDrvGetComMode

Prototype

Adpd400xComMode_t Adpd400xDrvGetComMode();

Description

Returns the communication bus: I2C, SPI or unknown

Parameters

None

Return value

Adpd400xComMode_t: ADPD400x_I2C_BUS, ADPD400x_SPI_BUS or ADPD400x_UNKNOWN_BUS

### Adpd400xDrvRegWrite

Prototype

int16_t Adpd400xDrvRegWrite(uint16_t nAddr, uint16_t nRegValue);

Description

This function does the synchronous register write of a 16-bit value to the device

Parameters

Name: nAddr

Type: uint16_t

Direction: Input/Output

Description: Register address to write

Name: nRegValue

Type: uint16_t

Direction: Input/Output

Description: Value to write to register

Return value

ADPD400xDrv_SUCCESS – Write to register successful

ADPD400xDrv_ERROR – Error in writing to register

### Adpd400xDrvRegRead

Prototype

int16_t Adpd400xDrvRegRead(uint16_t nAddr, uint16_t *pnData);

Description

This function does the synchronous register read of a 16-bit value from the device

Parameters

Name: nAddr

Type: uint16_t

Direction: Input/Output

Description: Register address to read

Name: pnData

Type: uint16_t *

Direction: Input/Output

Description: Pointer location to read the value into from register

Return value

ADPD400xDrv_SUCCESS – Read from register successful

ADPD400xDrv_ERROR – Error in reading register

### Adpd400xDrvRegRead32B

Prototype

int16_t Adpd400xDrvRegRead32B(uint16_t nAddr, uint32_t *pnData);

Description

This function does the synchronous register read of a 32-bit value from the device

Parameters

Name: nAddr

Type: uint16_t

Direction: Input/Output

Description: Register address to read

Name: pnData

Type: uint32_t *

Direction: Input/Output

Description: Pointer location to read the value into from register

Return value

ADPD400xDrv_SUCCESS – Read from register successful

ADPD400xDrv_ERROR – Error in reading register

### Adpd400xDrvSetOperationMode

Prototype

int16_t Adpd400xDrvSetOperationMode(uint8_t nOpMode);

Description

This function sets the operating mode of the device. And accordingly clears the FIFO.

Parameters

Name: nOpMode

Type: uint8_t

Direction: Input/Output

Description: Mode to set

Return value

ADPD400xDrv_SUCCESS – Setting mode successful

ADPD400xDrv_ERROR – Error in setting mode

### Adpd400xDrvSetOperationPause

Prototype

int16_t Adpd400xDrvSetOperationMode(uint8_t nEnable);

Description

This function sets the operating mode of the device to pause mode. And accordingly clears the FIFO.

Parameters

Name: nOpMode

Type: uint8_t

Direction: Input/Output

Description: Mode to set

Return value

ADPD400xDrv_SUCCESS – Setting pause mode successful

ADPD400xDrv_ERROR – Error in setting pause mode

### Adpd400xDrvSlotSetup

Prototype

int16_t Adpd400xDrvSlotSetup(uint8_t nSlotNum, uint8_t nEnable, uint16_t nSlotFormat, uint8_t nChannel);

Description

Sets up the slot for operation. The slot number starts from 0 for Slot A and ends with 11 for Slot L.

Parameters

Name: nSlotNum

Type: uint8_t

Direction: Input/Output

Description: Slot number to set

Name: eEnable

Type: uint8_t

Direction: Input/Output

Description: If this field is 0, then it disables all slots after the nSlotNum. If this field is 1, then it sets all slots

before the nSlotNum.

Name: nSlotFormat

Type: uint16_t

Direction: Input/Output

Description: Slot format for setting the data format in FIFO (Impulse, Dark, Sig)

Name: nChannel

Type: uint8_t

Direction: Input/Output

Description: Number of channels to enable

Return value

ADPD400xDrv_SUCCESS – Setting slot successful

ADPD400xDrv_ERROR – Error in setting slot

### Adpd400xDrvSlotSetActive

Prototype

int16_t Adpd400xDrvSlotSetActive(uint8_t nSlotNum, uint8_t nActive);

Description

Sets up a slot in sleep or active mode. The slot number starts from 0 for Slot A and ends with 11 for Slot L.

Parameters

Name: nSlotNum

Type: uint8_t

Direction: Input/Output

Description: Slot number to set

Name: nActive

Type: uint8_t

Direction: Input/Output

Description: If this field is 0, then it puts the slot into sleep. If this field is 1, then it sets the slot in awake

mode.

Return value

ADPD400xDrv_SUCCESS – Setting slot into sleep/active mode successful

ADPD400xDrv_ERROR – Error in setting slot into sleep/active modes

### Adpd400xDrvDataReadyCallback

Prototype

void Adpd400xDrvDataReadyCallback(void (*pfADPDDataReady)());

Description

Registers the data ready callback.

Parameters

Name: pfADPDDataReady

Type: void *

Direction: Input/Output

Description: Function pointer callback for the register data

Return value

None

### Adpd400xISR

Prototype

void Adpd400xISR();

Description

Interrupt service routine

Parameters

None

Return value

None

### Adpd400xDrvSetParameter

Prototype

int16_t Adpd400xDrvSetParameter(Adpd400xCommandStruct_t eCommand, uint8_t nPar, uint16_t nValue);

Description

Sets the configuration parameters for the device. The watermark for the FIFO is the option available now for configuring the device.

The test data command is used for internal testing only.

Parameters

Name: eCommand

Type: Adpd400xCommandStruct_t

Direction: Input/Output

Description: The command for the desired parameter to set

Name: nPar

Type: uint8_t

Direction: Input/Output

Description: This parameter is not used now

Name: nValue

Type: uint16_t

Direction: Input/Output

Description: The value to be set for the parameter

Return value

ADPD400xDrv_SUCCESS – Setting the parameter successful

ADPD400xDrv_ERROR – Error in setting the parameter

### Adpd400xDrvGetParameter

Prototype

int16_t Adpd400xDrvSetParameter(Adpd400xCommandStruct_t eCommand, uint8_t nPar, uint16_t *pnValue);

Description

Gets the configuration parameters for the device. The parameters that are obtained includes:

- Watermark

- Output Data Rate

- Fifo Level

- Time Gap

- Latest Slot data size

- Current slot data size

- Total slots data size

- If the current slot is active

- Check if this slot is selected

- Highest slot selected

- Number of active channels for this slot

Parameters

Name: eCommand

Type: Adpd400xCommandStruct_t

Direction: Input/Output

Description: The command for the desired parameter to get

Name: nPar

Type: uint8_t

Direction: Input/Output

Description: This parameter specifies the slot number

Name: pnValue

Type: uint16_t *

Direction: Input/Output

Description: The value to be set for the parameter

Return value

ADPD400xDrv_SUCCESS – Getting the parameter successful

ADPD400xDrv_ERROR – Error in getting the parameter

### Adpd400xDrvReadFifoData

Prototype

int16_t Adpd400xDrvReadFifoData(uint8_t *pnData, uint16_t nDataSetSize);

Description

This function does the read of data from the ADPD4100 FIFO

Parameters

Name: pnData

Type: uint8_t *

Direction: Input/Output

Description: Pointer location to read data into

Name: nDataSetSize

Type: uint16_t

Direction: Input/Output

Description: Dataset size to read

Return value

ADPD400xDrv_SUCCESS – Read from FIFO successful

ADPD400xDrv_ERROR – Error in reading from FIFO

### Adpd400xDrvReadRegData

Prototype

int16_t Adpd400xDrvReadRegData(uint32_t *pnData, ADPD400xDrv_SlotNum_t nSlotNum, uint8_t nSignalDark, uint8_t nChNum);

Description

This function does the read of data from the ADPD4100 register for a particular slot

Parameters

Name: pnData

Type: uint32_t *

Direction: Input/Output

Description: Pointer location to read data into

Name: nSlotNum

Type: ADPD400xDrv_SlotNum_t

Direction: Input/Output

Description: Slot number to read data from

Name: nSignalDark

Type: uint8_t

Direction: Input/Output

Description: Dark/Signal flag

Name: nChNum

Type: uint8_t

Direction: Input/Output

Description: Channel number from the slot to read

Return value

ADPD400xDrv_SUCCESS – Read from register of a slot successful

ADPD400xDrv_ERROR – Error in reading from register for a slot

### Adpd400xDrvSetLedCurrent

Prototype

int16_t Adpd400xDrvSetLedCurrent(uint16_t nLedCurrent, ADPD400xDrv_LedId_t nLedId, ADPD400xDrv_SlotNum_t nSlotNum);

Description

This function sets the LED current for the LED current to the slot

Parameters

Name: nLedCurrent

Type: uint16_t

Direction: Input/Output

Description: LED current to set to. 0: disable, max is 0x7F = 200mA (for details of this setting, see DataSheet)

Name: nLedId

Type: ADPD400xDrv_LedId_t

Direction: Input/Output

Description: LED number. Check the LED connected to each slot on the platform

Name: nSlotNum

Type: ADPD400xDrv_SlotNum_t

Direction: Input/Output

Description: Slot Number of the LED

Return value

ADPD400xDrv_SUCCESS – Setting LED current successful

ADPD400xDrv_ERROR – Error in setting LED current

### Adpd400xDrvGetLedCurrent

Prototype

int16_t Adpd400xDrGetLedCurrent(uint16_t *pLedCurrent, ADPD400xDrv_LedId_t nLedId, ADPD400xDrv_SlotNum_t nSlotNum);

Description

This function sets the LED current for the LED current to the slot

Parameters

Name: pLedCurrent

Type: uint16_t

Direction: Input/Output

Description: Pointer location to get LED current

Name: nLedId

Type: ADPD400xDrv_LedId_t

Direction: Input/Output

Description: LED number. Check the LED connected to each slot on the platform

Name: nSlotNum

Type: ADPD400xDrv_SlotNum_t

Direction: Input/Output

Description: Slot Number of the LED

Return value

ADPD400xDrv_SUCCESS – Getting LED current successful

ADPD400xDrv_ERROR – Error in getting LED current

### Adpd400xDrvSoftReset

Prototype

int16_t Adpd400xDrvSoftReset(void);

Description

This function does a soft reset of the ADPD4100 device

Parameters

None

Return value

ADPD400xDrv_SUCCESS – Soft reset of ADPD4100 successful

ADPD400xDrv_ERROR – Error in soft resetting of ADPD4100

### _Adpd400xDrvInit

Prototype

static void _Adpd400xDrvInit(void);

Description

This function initializes the driver.

Parameters

None

Return value

None

### _Adpd400xDrvSetInterrupt

Prototype

static int16_t _Adpd400xDrvSetInterrupt();

Description

This function sets the FIFO interrupt mode.

Parameters

None

Return value

ADPD400xDrv_SUCCESS – Setting of FIFO interrupt mode successful

ADPD400xDrv_ERROR – Error in setting FIFO interrupt mode

### _Adpd400xDrvSetIdleMode

Prototype

static int16_t _Adpd400xDrvSetIdleMode();

Description

This function sets the device to Idle mode.

Parameters

None

Return value

ADPD400xDrv_SUCCESS – Setting to Idle mode successful

ADPD400xDrv_ERROR – Error in setting to Idle mode

### _Adpd400xDrvGetSlotInfo

Prototype

static void _Adpd400xDrvGetSlotInfo();

Description

This function gets the status of the twelve slots.

Parameters

None

Return value

ADPD400xDrv_SUCCESS – Getting the status of slots successful

ADPD400xDrv_ERROR – Error in getting the status of slots

### _Adpd400xDrvGetDataOuputRate

Prototype

static void _Adpd400xDrvGetDataOuputRate();

Description

This function gets the output data rate.

Parameters

None

Return value

ADPD400xDrv_SUCCESS – Getting the output data rate successful

ADPD400xDrv_ERROR – Error in getting the output data rate

### _Adpd400xDrvSlotSaveCurrentSetting

Prototype

static void _Adpd400xDrvSlotSaveCurrentSetting(uint8_t nSlotNum);

Description

This function saves the current setting

Parameters

Name: nSlotNum

Type: uint8_t

Direction: Input/Output

Description: Slot Number of the LED

Return value

ADPD400xDrv_SUCCESS – Saving the current setting successful

ADPD400xDrv_ERROR – Error in saving current setting

### _Adpd400xDrvSlotApplyPreviousSetting

Prototype

static void _Adpd400xDrvSlotApplyPreviousSetting(uint8_t nSlotNum);

Description

This function is used to apply the previous LED settings

Parameters

Name: nSlotNum

Type: uint8_t

Direction: Input/Output

Description: Slot Number of the LED

Return value

ADPD400xDrv_SUCCESS – Applying the previous LED settings successful

ADPD400xDrv_ERROR – Error in applying the previous LED settings

### _Adpd400xDrvSlotApplySkipSetting

Prototype

static void _Adpd400xDrvSlotApplySkipSetting(uint8_t nSlotNum);

Description

This function restores the previous LED settings

Parameters

Name: nSlotNum

Type: uint8_t

Direction: Input/Output

Description: Slot Number of the LED

Return value

ADPD400xDrv_SUCCESS – Restoring the previous LED settings successful

ADPD400xDrv_ERROR – Error in restoring the previous LED settings

### _Adpd400xDrvSetSlotSize

Prototype

static void _Adpd400xDrvSetSlotSize(uint8_t nSlotNum, uint16_t nSlotFormat);

Description

This function sets the slot data size

Parameters

Name: nSlotNum

Type: uint8_t

Direction: Input/Output

Description: Slot Number of the LED

Name: nSlotFormat

Type: uint16_t

Direction: Input/Output

Description: Slot Format of the data

Return value

ADPD400xDrv_SUCCESS – Setting the slot data size successful

ADPD400xDrv_ERROR – Error in setting the slot data size

### _FifoLevel

Prototype

static uint16_t _FifoLevel(void;

Description

This function finds the FIFO level

Parameters

None

Return value

Fifo level

## API Data Types

### adpd400xDrv_slot_t

typedef struct _adpd400xDrv_slot_t {

uint8_t  activeSlot;      //!< Active slot

uint8_t  pre_activeSlot;  //!< Previous Active slot

uint16_t slotFormat;      //!< Dark,Sig,Lit,Ttl bytes of each slot

uint8_t  channelNum;      //!< Active channel for each slot

uint8_t  decimation;

} adpd400xDrv_slot_t;

Description

The adpd400xDrv_slot_t structure contains the various parameters related to a slot of ADPD4100.

Fields

- adpd400xDrv_slot_t.activeSlot specifies if the slot is active

- adpd400xDrv_slot_t.pre_activeslot stores the previous active slot

- adpd400xDrv_slot_t.slotFormat specifies the format of the slot data (dark/sig/lit)

- adpd400xDrv_slot_t.channelNum indicates the active the channels for the slot

- adpd400xDrv_slot_t.decimation indicates the decimation factor for the slot

## Enums

### ADPD400xDrv_Operation_Mode_t

typedef enum {

ADPD400xDrv_MODE_IDLE = 0,

ADPD400xDrv_MODE_PAUSE,

ADPD400xDrv_MODE_PWR_OFF,

ADPD400xDrv_MODE_SAMPLE

} ADPD400xDrv_Operation_Mode_t;

Description

Indicates the operation modes of the ADPD4100 device

### ADPD400XDrv_FIFO_SIZE_t

typedef enum {

ADPD400xDrv_SIZE_0  = 0x00,

ADPD400xDrv_SIZE_8  = 0x01,

ADPD400xDrv_SIZE_16 = 0x02,

ADPD400xDrv_SIZE_24 = 0x03,

ADPD400xDrv_SIZE_32 = 0x04,

} ADPD400XDrv_FIFO_SIZE_t;

Description

Enumerates the various data sizes of the FIFO data. This can be a byte as minimum and 4 bytes as maximum.

### Adpd400xCommandStruct_t

typedef enum {

ADPD400x_WATERMARKING = 0,

ADPD400x_FIFOLEVEL,

ADPD400x_OUTPUTDATARATE,

ADPD400x_TIMEGAP,

ADPD400x_LATEST_SLOT_DATASIZE,

ADPD400x_THIS_SLOT_DATASIZE,

ADPD400x_SUM_SLOT_DATASIZE,

ADPD400x_IS_SLOT_ACTIVE,

ADPD400x_IS_SLOT_SELECTED,

ADPD400x_HIGHEST_SLOT_NUM,

ADPD400x_THIS_SLOT_CHANNEL_NUM,

ADPD400x_TEST_DATA

} Adpd400xCommandStruct_t;

Description

This shows the various commands used for setting or getting parameters of the ADPD4100 device.

### ADPDDrvCl_SignalDark_t

typedef enum {

ADPD400xDrv_SIGNAL = 0x00,

ADPD400xDrv_DARK

} ADPDDrvCl_SignalDark_t;

Description

This structure lists the signal and dark parts of the data

### ADPD400xDrv_SlotNum_t

typedef enum {

ADPD400xDrv_SLOTA = 0x00,

ADPD400xDrv_SLOTB,

ADPD400xDrv_SLOTC,

ADPD400xDrv_SLOTD,

ADPD400xDrv_SLOTE,

ADPD400xDrv_SLOTF,

ADPD400xDrv_SLOTG,

ADPD400xDrv_SLOTH,

ADPD400xDrv_SLOTI,

ADPD400xDrv_SLOTJ,

ADPD400xDrv_SLOTK,

ADPD400xDrv_SLOTL

} ADPD400xDrv_SlotNum_t;

Description

This structure maps the slots on the ADPD4100 device

### ADPD400xDrv_LedId_t

typedef enum {

ADPD400xDrv_LED_OFF = 0x00,

ADPD400xDrv_LED1,

ADPD400xDrv_LED2,

ADPD400xDrv_LED3,

ADPD400xDrv_LED4

} ADPD400xDrv_LedId_t;

Description

This structure maps the LEDs available on the ADPD4100 device

### Adpd400xComMode_t

typedef enum {

ADPD400x_I2C_BUS,    /**< enum value 0 */

ADPD400x_SPI_BUS,    /**< enum value 1 */

ADPD400x_UNKNOWN_BUS /**< enum value 2 */

} Adpd400xComMode_t;

Description

This enumeration shows the different data interface modes available on ADPD410X devices